PEAR SORTING VISION SYSTEM

Technical Code Report & Configuration Guide

Real-Time Pear Quality Inspection Pipeline

# 1. System Overview

This system is a real-time, industrial-grade computer vision pipeline designed to inspect and sort pears on a conveyor belt. A single camera captures frames of a tray divided into 6 fixed positions (ROIs). Each pear is independently analyzed for presence and surface infection, and a corresponding servo motor either accepts or rejects it.

## 1.1 Pipeline at a Glance

The system processes one camera frame per second (configurable) through 7 sequential tasks:

- Frame Capture → Camera reads a full BGR frame

- Frame Division → Frame split into 6 ROI sub-images

- Preprocessing → Median blur + HSV conversion per ROI

- Pear Detection → HSV threshold + morphology + contour validation

- Infection Detection → Brown/dark pixel detection inside pear mask

- Feature Packing → All data merged into 6-value output vectors

- Motor Task → ACCEPT/REJECT decisions sent to servo actuators

## 1.2 Target Hardware

- Platform: NVIDIA Jetson Nano (or Raspberry Pi 4)

- Camera: USB webcam or CSI camera (OpenCV compatible)

- Motor Controller: PCA9685 I²C servo driver (6 channels)

- Servos: Standard PWM servos, 0° = accept, 180° = reject

# 2. Module Structure

The codebase follows an industrial single-responsibility architecture. Each module handles one task and passes structured data objects - never raw images - to the next stage.

| File | Responsibility | Key Functions |
| --- | --- | --- |
| config.py | All tunable system parameters (the only file you should need to edit during deployment calibration) | - (constants only) |
| main.py | System entry point, main real-time loop, pipeline orchestration | main(), run_pipeline(), parse_args() |
| frame_divider.py | Splits full camera frame into 6 ROI sub-images using pixel coordinates from config | frame_divider(), visualize_rois() |
| preprocessing.py | Applies median blur and BGR→HSV conversion to each ROI | preprocess_roi(), preprocess_all_rois() |
| pear_detection.py | HSV threshold → morphology → contour extraction → shape validation per ROI | pear_detection(), detect_all_pears() |
| infection_detection.py | Detects brown/dark/infected pixels inside the pear mask; computes area, hue, and location | infection_detection(), detect_all_infections() |
| feature_extraction.py | Packs all detection results into the standard 6-value output vector | format_roi_output(), format_frame_output() |
| motor_task.py | Translates feature vectors into motor angle commands via decision and action mapping | motor_task(), decision_function(), action_mapper(), motor_command() |
| publisher.py | Sends final frame data to print, file, ROS2, or serial depending on PUBLISH_MODE | publish() |
| utils.py | Camera init/release, debug visualization overlays, loop timer, test image loader | init_camera(), get_frame(), draw_debug_overlay() |

# 3. Output Format

## 3.1 Per-ROI Feature Vector

Every ROI produces a 6-element list that fully describes the pear state in that slot:

| Index | Name | Values | Description |
| --- | --- | --- | --- |
| 0 | pear_flag | 0 or 1 | 1 = pear detected in this ROI; 0 = empty slot |
| 1 | quality_flag | 0 or 1 | 1 = healthy pear (ACCEPT); 0 = infected (REJECT) |
| 2 | infection_hue | 0.0 – 179.0 | Mean HSV hue value of the infected region |
| 3 | infection_area | float cm² | Total infected surface area (converted via SCALE_FACTOR) |
| 4 | pear_area | float cm² | Total pear surface area (converted via SCALE_FACTOR) |
| 5 | infection_location | 0/1/2/3 | 0=top, 1=middle, 2=bottom third of pear; 3=no infection |

## 3.2 Full Frame Output

One complete frame produces an array of 6 vectors (one per ROI), in the order defined by ROI_ORDER in config.py:

Frame_Data = [

[pear_flag, quality_flag, inf_hue, inf_area, pear_area, inf_loc],  # ROI A3

[pear_flag, quality_flag, inf_hue, inf_area, pear_area, inf_loc],  # ROI B3

[pear_flag, quality_flag, inf_hue, inf_area, pear_area, inf_loc],  # ROI A2

[pear_flag, quality_flag, inf_hue, inf_area, pear_area, inf_loc],  # ROI B2

[pear_flag, quality_flag, inf_hue, inf_area, pear_area, inf_loc],  # ROI A1

[pear_flag, quality_flag, inf_hue, inf_area, pear_area, inf_loc],  # ROI B1

]

# 4. Configuration Guide (config.py)

All tunable parameters live in config.py. This section explains every variable, its default value, and the effect of adjusting it. You should ONLY need to modify config.py during calibration - all other files read from it.

## 4.1 Camera Settings

| Parameter | Default | Effect of Increasing | Effect of Decreasing |
| --- | --- | --- | --- |
| CAMERA_INDEX | 0 | Selects a higher-numbered camera device | Selects a lower-numbered or different camera |
| FRAME_WIDTH | 1280 | Higher resolution → more detail but slower processing | Lower resolution → faster but less fine detection |
| FRAME_HEIGHT | 720 | Higher → more vertical resolution | Lower → faster processing |
| LOOP_SLEEP_SEC | 1.0 | Slower frame rate → less CPU load | Faster frame rate → more responsive sorting |

## 4.2 Pear Detection - HSV Color Thresholds

These are the most critical parameters. Incorrect HSV bounds are the #1 cause of missed or false pear detections.

| Parameter | Default | Effect of Increasing | Effect of Decreasing |
| --- | --- | --- | --- |
| PEAR_HSV_LOWER[0] (H) | 15 | Shifts toward more yellow-green (excludes orange-red) | Includes more orange-red hues; may catch background |
| PEAR_HSV_LOWER[1] (S) | 30 | Requires more vivid color; rejects pale backgrounds | Accepts less saturated (paler) colors; more noise |
| PEAR_HSV_LOWER[2] (V) | 60 | Excludes darker regions; cleaner mask in low light | Includes shadow areas; may merge pear with shadow |
| PEAR_HSV_UPPER[0] (H) | 95 | Covers less hue range; may miss yellowish pears | Covers more range; may include non-pear objects |
| PEAR_HSV_UPPER[2] (V) | 255 | No change at max; lower to exclude bright highlights | Excludes overexposed reflections (lower to ~200) |

## 4.3 Pear Detection - Morphology Kernels

| Parameter | Default | Effect of Increasing | Effect of Decreasing |
| --- | --- | --- | --- |
| MORPH_OPEN_KERNEL | 7×7 | Removes larger noise blobs; may erode pear edges | Preserves fine detail but keeps more noise |
| MORPH_CLOSE_KERNEL | 15×15 | Fills larger internal holes (e.g., big dark spots) | Preserves shape accuracy; may leave small holes |

## 4.4 Pear Detection - Validation Thresholds

| Parameter | Default | Effect of Increasing | Effect of Decreasing |
| --- | --- | --- | --- |
| PEAR_MIN_AREA | 3000 px² | Rejects more small false detections | Detects smaller/partial pears; more noise accepted |
| PEAR_MAX_AREA | 300000 px² | Rejects oversized merged blobs more aggressively | Accepts larger blobs; risk of accepting background |
| PEAR_MIN_ASPECT_RATIO | 0.4 | Rejects more elongated (thin) shapes | Accepts more elongated shapes |
| PEAR_MAX_ASPECT_RATIO | 1.6 | Rejects wider/flatter shapes | Accepts wider shapes; may catch flat objects |
| PEAR_MIN_SOLIDITY | 0.6 | Requires more convex shapes; stricter | Accepts irregular or concave shapes |

## 4.5 Infection Detection - Color Thresholds

| Parameter | Default | Effect of Increasing | Effect of Decreasing |
| --- | --- | --- | --- |
| INFECTION_HSV_UPPER[0] (H) | 30 | Narrows to darker browns only | Wider range; may catch healthy orange-toned skin |
| INFECTION_HSV_UPPER[2] (V) | 100 | Only very dark infections detected | Lighter brown infections also detected; more false positives |
| INFECTION_DARK_V_MAX | 60 | Only nearly-black pixels flagged | More dark regions flagged; may include shadows |

## 4.6 Infection Quality Decision Thresholds

| Parameter | Default | Effect of Increasing | Effect of Decreasing |
| --- | --- | --- | --- |
| INFECTION_AREA_THRESHOLD | 500 px² | Stricter: smaller infections → REJECT | Looser: requires larger infections to reject |
| INFECTION_RATIO_THRESHOLD | 0.05 (5%) | Stricter: smaller infection fraction → REJECT | Looser: allows more infection coverage before rejecting |
| SCALE_FACTOR | 0.01 | Areas reported as larger (camera zoomed in) | Areas reported as smaller (camera zoomed out) |

## 4.7 Motor Settings

| Parameter | Default | Effect of Increasing | Effect of Decreasing |
| --- | --- | --- | --- |
| MOTOR_ACCEPT_ANGLE | 0° | Changes gate angle for accept lane | Changes gate angle for accept lane |
| MOTOR_REJECT_ANGLE | 180° | Changes gate angle for reject bin | Changes gate angle for reject bin |
| PUBLISH_MODE | "print" | More permanent outputs (file/ros2/serial) | Simpler outputs for development |

# 5. Decision & Motor Logic

## 5.1 Decision Function

For each ROI, the decision_function() applies these rules in order:

if pear_flag == 0:  → NO_ACTION   (slot is empty)

elif quality_flag == 1:  → ACCEPT  (motor → 0°)

else:  → REJECT              (motor → 180°)

This design is fully stateless - no memory of previous frames. Every frame stands alone, which prevents cascading errors.

## 5.2 Motor Behavior Table

| pear_flag | quality_flag | Decision | Servo Angle |
| --- | --- | --- | --- |
| 0 (no pear) | - | NO_ACTION | No movement |
| 1 (pear present) | 1 (good) | ACCEPT | 0° (accept lane) |
| 1 (pear present) | 0 (infected) | REJECT | 180° (reject bin) |

# 6. Calibration Workflow

## Step 1 - Camera Alignment

- Run python main.py --debug to open the live debug window

- Adjust the physical camera position until all 6 ROI rectangles align with tray slots

- Update ROI_DEFINITIONS in config.py to match observed positions

## Step 2 - Pear HSV Tuning

- Capture a frame with known healthy pears in all slots

- Use cv2.inRange() manually or run the visualize_rois() helper to see the mask

- Adjust PEAR_HSV_LOWER/UPPER until all pear bodies are white in the mask with no background leaking

## Step 3 - Infection Threshold Tuning

- Use infected pear samples (like images 7–16 in the reference set)

- Adjust INFECTION_HSV_UPPER[2] (V channel): if light infections are missed, increase toward 120–130

- Adjust INFECTION_AREA_THRESHOLD: if healthy pears with small spots are wrongly rejected, increase

- Adjust INFECTION_RATIO_THRESHOLD: for stricter quality control, decrease toward 0.02–0.03

## Step 4 - Scale Factor Calibration

- Place an object of known size (e.g., 10×10 cm paper) in one ROI

- Run the system and note the reported pear_area

- Set SCALE_FACTOR = real_area_cm2 / reported_pixel_area

## Step 5 - Motor Angle Calibration

- Set PUBLISH_MODE = 'print' and observe decisions

- Manually test MOTOR_ACCEPT_ANGLE and MOTOR_REJECT_ANGLE to confirm physical gate positions

- Assign correct MOTOR_CHANNELS for each tray slot's servo wire

# 7. Running the System

## 7.1 Installation

pip install opencv-python numpy

pip install adafruit-circuitpython-pca9685 adafruit-circuitpython-motor  # for real hardware

## 7.2 Run Modes

Live camera + debug window

python main.py --debug

Test with a static pear image

python main.py --test --image /path/to/pear_image.jpg

Live camera, silent output to file

# In config.py: PUBLISH_MODE = 'file'

python main.py

## 7.3 Output Example

When PUBLISH_MODE = 'print', each frame prints:

[FRAME 12]

[A3] PEAR | GOOD     | PearArea=42.3cm² | InfArea=0.12cm² | InfHue=22.1 | InfLoc=NONE

[B3] PEAR | INFECTED | PearArea=38.7cm² | InfArea=6.80cm² | InfHue=14.3 | InfLoc=TOP

[A2] EMPTY | ...

[MOTOR SIM] Channel 0 → 0°    (A3 ACCEPT)

[MOTOR SIM] Channel 1 → 180°  (B3 REJECT)

# 8. Design Rules & Principles

## Rule 1 - One Responsibility Per Module

No module performs more than its single assigned task. pear_detection.py never decides motor angles. motor_task.py never opens an image.

## Rule 2 - No Cross-Task Logic

Each task receives only what it needs and outputs only its defined data structure. There are no global image arrays modified by multiple modules.

## Rule 3 - Stateless Frames

Every frame is processed independently. The motor task does not remember what happened in previous frames. This makes the system deterministic and easy to debug.

## Rule 4 - Independent ROIs

Each of the 6 ROIs is treated as a miniature independent system. This enables parallel processing, per-slot debugging, and easy scaling to more camera positions.

## Rule 5 - All Tuning via config.py

No magic numbers appear inside the pipeline code. Every threshold, kernel size, angle, or limit references a named constant from config.py with an explanation of its effect.

- End of Report -